# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

- Contract title: Framework Contract for Supply and delivery of automotive tires for EUMM vehicles fleet	p 1 / 7

Publication reference: EUMM-24-9364

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to precisely identify the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

| 1. Item Number | 2. Specifications Required | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| 1 Renault Duster | Tire 215/60 R17 - Weather: M&S/All season - Load index single use: minimum 80 - Speed index: minimum S - Tubeless - ECE Marked - Production Year: - For Dec 2024-Dec 2026 FWC: DOT after 2524 - For Dec 2026-Dec 2028 FWC: DOT after 2526 Estimated Quantity: 10 |  |  |  |
| 2 Renault Duster | Tire 215/60 R17 - Weather: M&S/ 3PMSF - Load index single use: minimum 80 - Speed index: minimum S - Tubeless - ECE Marked - Production Year: - For Dec 2024-Dec 2026 FWC: DOT after 2524 - For Dec 2026-Dec 2028 FWC: DOT after 2526 Estimated Quantity: 10 |  |  |  |
| 3 Mercedes Vito | Tire 225/55 R17 C - Weather: M&S/All season - Load index single use: minimum 80 - Speed index: minimum R - Tubeless - ECE Marked - Production Year: - For Dec 2024-Dec 2026 FWC: DOT after 2524 - For Dec 2026-Dec 2028 FWC: DOT after 2526 Estimated Quantity: 10 |  |  |  |
| 4. Toyota LC76 HS/SS | Tire 235/85 R16 - Weather: M&S/Mountain snowflake symbol marked - All terrain type tread/AT marked - Load index single use: minimum 112 - Speed index: minimum R - Tubeless - ECE Marked - Production Year: - For Dec 2024-Dec 2026 FWC: DOT after 2524 - For Dec 2026-Dec 2028 FWC: DOT after 2526 Estimated Quantity: 10 |  |  |  |
| 5 Ford Transit | Tire 215/75 R16 C - Weather: M&S/Mountain snowflake symbol marked - Load index single use: minimum 112 - Speed index: minimum R - Tubeless - ECE Marked - Production Year: - For Dec 2024-Dec 2026 FWC: DOT after 2524 - For Dec 2026-Dec 2028 FWC: DOT after 2526 Estimated Quantity: 10 |  |  |  |
| 6 Toyota Prado/ Hilux | Tire 265/65 R17 - Weather: M&S/Mountain snowflake symbol marked- 3PMSF - Load index single use: minimum 88 - Speed index: minimum S - Soft pattern tread for winter - Tubeless - ECE Marked - Production Year: - For Dec 2024-Dec 2026 FWC: DOT after 2524 - For Dec 2026-Dec 2028 FWC: DOT after 2526 Estimated Quantity: 10 |  |  |  |
| 7 Toyota Prado/ Hilux | Tire 265/65 R17 - Weather: Summer type - Load index single use: minimum 88 - Speed index: minimum S - Tubeless - ECE Marked - Production Year: - For Dec 2024-Dec 2026 FWC: DOT after 2524 - For Dec 2026-Dec 2028 FWC: DOT after 2526 Estimated Quantity: 10 |  |  |  |
| 8 Mercedes G class | Tire 265/70 R16 - Weather: M&S/Mountain snowflake symbol marked 3PMSF - All Terrain Tread Design - Load index single use: minimum 115 - Speed index: minimum Q - Tubeless - ECE Marked - Production Year: - For Dec 2024-Dec 2026 FWC: DOT after 2524 - For Dec 2026-Dec 2028 FWC: DOT after 2526 Estimated Quantity: 10 |  |  |  |
| 9 Man TGM | TIRE 295/80 R22.5 - Weather: M&S/All season - Load index single use: minimum 152/148 - Speed index: minimum K - Tubeless - ECE Marked - Production Year: - For Dec 2024-Dec 2026 FWC: DOT after 2524 - For Dec 2026-Dec 2028 FWC: DOT after 2526 Estimated Quantity: 10 |  |  |  |
| 10. Toyota Coaster RR | TIRE 7.00 R 16 - Weather: M&S/All season - Load index single use: minimum 117/116 - Speed index: minimum L - Tube - ECE Marked - Production Year: - For Dec 2024-Dec 2026 FWC: DOT after 2524 - For Dec 2026-Dec 2028 FWC: DOT after 2526 Estimated Quantity: 10 |  |  |  |
| 11. Toyota Coaster FR | TIRE 7.00 R16 LSR - Weather: M&S/All season - Load index single use: minimum 117/116 - Speed index: minimum L - Tube - ECE Marked - Production Year: - For Dec 2024-Dec 2026 FWC: DOT after 2524 - For Dec 2026-Dec 2028 FWC: DOT after 2526 Estimated Quantity: 10 |  |  |  |
| 12 URO Vamtac | Tire 335/80 R20 MPT 81 - Weather: M&S/All season - Load index single use: minimum 147 - Speed index: minimum K - Tubeless - ECE Marked - Production Year: - For Dec 2024-Dec 2026 FWC: DOT after 2524 - For Dec 2026-Dec 2028 FWC: DOT after 2526 Estimated Quantity: 10 |  |  |  |
| 13 Skid car | Tire 15X4 ½-8 ind.12PR 100A5 IC 36 - Speed index A5 - Load index: min 90 - ECE Marked - Production Year: - For Dec 2024-Dec 2026 FWC: DOT after 2524 - For Dec 2026-Dec 2028 FWC: DOT after 2526 Estimated Quantity: 5 |  |  |  |

***